Welding and Cutting

# Company Policy

Company Name is dedicated to the protection of its employees from occupational injuries and illnesses.  Company Name is responsible for providing a safe working environment, and the employees have and assume the responsibility of working safely.

The objective of this program is to supplement the safety policy by providing specific standards regarding Welding and Cutting procedures, and to ensure that each employee is adequately trained and fully aware of safety procedures associated with welding and cutting.

Elimination of injuries and illnesses improves employee morale, improves customer service, improves product quality, and reduces Workers’ Compensation costs.

Job Title/Specific Name will conduct routine safety inspections of jobsites to ensure compliance with this program.  Job Title/Specific Name has the authority to enforce the Welding and Cutting policy in accordance to any and all Company Name safety rules and applicable OSHA regulations.

Employees are required to comply with the guidelines set forth, and to comply with the instruction of Job Title/Specific Name.  In the event an unsafe condition arises in the absence of Job Title/Specific Name, employees shall alert the lead person on the jobsite immediately.  Employees shall alert coworkers of any unsafe conditions that arise.

Any Company Name employee who disobeys and/or disregards the guidelines set forth in this program or the company’s safety program shall be subject to disciplinary action.

### Transporting, Moving and Storage

- Valve caps on cylinders shall be in place and secured. Valve caps shall not be used for lifting.  Do not pry cylinder caps while frozen.  Loosen caps with warm water.

- Cylinders shall be transported on a secured cradle only, and by tilting or rolling them.

- Cylinders shall be moved by tilting and rolling them on their bottom edges. Avoid dropping cylinders or striking other cylinders.

- Cylinders shall be transported by powered vehicles shall be secured in a vertical position.

- Regulators shall be removed and caps put in place prior to moving cylinders, unless cylinders are secured on a special carrier.

- Proper steadying devices shall be used to keep cylinders from falling over while in use.

- Cylinder valves shall be closed when cylinders are empty or when cylinders are moved.

- Oxygen cylinders shall be stored separated from fuel gas cylinders or combustible materials a minimum distance of 20 feet or by a 5 foot high noncombustible barrier with a fire-resistance rating of one-half hour.

- Cylinders stored inside buildings shall be stored 20 feet from combustible materials where they are well protected, well ventilated, and dry.  Cylinders shall not be stored near elevators, stairs or gangways.  Assigned storage locations shall prevent cylinders from being knocked over or damaged.

Placing cylinders.

- Cylinders shall be kept away from welding or cutting operations to prevent sparks, hot slag, or flame from reaching them. Fire resistant shields shall be used when this is impractical.

- Cylinders shall be placed away from electrical circuits. Do not strike electrodes against a cylinder to strike an arc.

- Cylinders containing oxygen, acetylene or other fuel gas shall not be used in confined spaces.

### Treatment of Cylinders

- Cylinders shall not be used as rollers or supports.

- Only the gas supplier is authorized to mix gases in a cylinder.  Only Job Title/Specific Name is required to have cylinders refilled.

- No damaged or defective cylinder shall be used.

### Use of Fuel Gas

- Company Name employees shall be instructed in the safe use of fuel gas.

- Valves shall be opened slightly and closed immediately before a regulator is connected to the cylinder.  This is called “cracking” which clears the valve of dust and dirt.  The employee shall stand to the side of the outlet, not in front.  Valves shall be cracked away from welding work, sparks, flames or other sources of ignition.

- Valves shall be opened slowly to prevent damage to the regulator.  Valves shall not be opened more than 1 ½ turns. If a wrench is required it shall stay in position in case of emergency for a quick shut off.  Manifolded or coupled cylinders shall have a wrench available for immediate use.  Do not place objects on top of cylinders, or damage may occur to the safety device or interfere with the quick closing of the valve.

- Cylinders shall be closed and the gas released from the regulator before removing the regulator.

- If cylinders, valves, regulators, plugs, or other safety devices are damaged, they shall be tagged out of service and removed from the work area.

Manifolds

- Fuel gas and oxygen manifolds shall bear the name of the substance they contain.

- Fuel gas and oxygen manifolds shall not be placed in confined spaces, but shall be placed in safe, well ventilated, and accessible locations.

- Hose connections shall work that they cannot be interchanged between fuel gas and oxygen manifolds and supply header connections.  Keep hose connection free of grease and oil, and do not use adapters to interchange hoses.

- Manifold and header hose connections shall be capped when not in use.

- Nothing shall be placed on manifolds that will damage the manifold or interfere with the quick closing of the valves.

### Hoses

- Fuel gas hose and oxygen hose shall be distinguishable from each other, and not interchangeable.  Contrast may be made by different colors or by surface characteristics distinguishable by the sense of touch.

- All hose in use shall be inspected at the beginning of each work shift by employees.  Defective hose shall be tagged and removed from service.

- Hose subjected to flashback, or which show severe wear or damage must be tested twice the normal pressure for the hose, but not less than 300 p.s.i.  If defective, the hose shall not be used.

- Hose couplings shall be disconnected by rotary motion only.

- Hose stored in boxes shall be well ventilated.  Hoses, cables, and other equipment shall be kept clear of passageways, ladders and stairs.

Torches.

- Torches shall be inspected at the beginning of each working shift for leaking shutoff valves, hose couplings, and tip connections. Defective torches shall not be used.

- Clogged torch tip openings shall be cleaned.

- Torches shall be lit by friction lighters or other approved devices only.

Regulators and gauges.

- Pressure regulators and related gauges, shall work properly while in use.

Oil and grease hazards.

- Oxygen cylinders and fittings shall be kept away from oil or grease. Cylinders and fittings shall be kept free from oil or greasy substances and shall not be handled with oily hands or gloves. Oxygen shall not be directed at oily surfaces, greasy clothes, or

within a fuel oil or other storage tank or vessel.

## Fire Protection

- Objects to be welded, cut or heated shall be moved to a designated safe location.  If the object cannot be easily moved, all moveable fire hazards shall be moved or protected.

- If the object to be welded, cut, or heated cannot be moved and if all the fire hazards cannot be removed, positive means shall be taken to confine the heat, sparks, and slag, and to protect the immovable fire hazards from them.

- Welding, cutting, or heating shall not be performed in the presence of flammable paints, flammable compounds or heavy dust concentrations.

- Fire extinguishers shall be immediately available in the work area, free of obstruction, and maintained for instant use.

- When normal fire prevention precautions are not sufficient for the welding, cutting, or heating operation Job Title/Specific Name shall be assigned as a fire watch. Sufficient amount of time shall be allowed after completion of work to ensure that the possibility of fire does not exist.  The designated fire watch shall be trained in firefighting equipment.

- Gas supplies shall be shut off during lunch breaks, overnight, or during shift breaks.  Hoses and torches shall be removed from confined spaces.

## Training

- Company Name employees are required to become familiar with and understand the guidelines regarding Welding and Cutting operations.  Designated welders and cutters must receive annual training and must demonstrate their understanding of these guidelines to Job Title/Specific Name.

- Upon completion of Welding and Cutting training, Company Name shall certify in writing that each designated flagman has received and understands training requirements.  Certification shall include the employee’s name, date of training, and subject of certification.

## Conclusion

All employees of Company Name are required to comply with the rules set forth in this written program.  This program is intended to provide the maximum protection for employees of Company Name.